# CSO Placement

## Important features

Spills are persistent  rules out rainfall shock variations

Spills are larger than anyone knew  information value in CSO rollout

## Potential instruments for CSO placement

- Rollout of EDMs

Observed spills affected by EDM activation and other factors, news stories if possible at local level

Non-random rollout prioritising ecological sites like bathing waters and SSSIs, and possibly falsify where pressure groups were located.

- Geological/natural geographical features

Catchment slope, Soil infiltration capacity and Drainage density.

Downstream v upstream of CSOs (need elevation and water flow maps). Perrmeability of soil so how much absorbed.

Drainage density of how many tributaries the sewer system had to consolidate. Predicted spills more likely in downstream locations or in areas with upstream impermeable land or more network junctions.

- Historic hydraulic features

Treatment Plants

Historically determined and highly persistent, and anchor drainage direction of catchment areas.

- Distance to water works treatment plants  explains pressure on system. Could be sited near industry so downstream areas may have historic trajectories.

- Closures of treatment plants  may have arisen due to pollution concerns but this should not affect where pressure relief is mechanically required in the system (instead affects where treatment eventually occurs unless new capacity added). More affected by closures when have low elevation within catchment (Stream Portal has maps), have capacity constraints and downstream tailwater conditions.

- Upgrades  Discrete, large-scale interventions that reduce spill frequency for specific CSO catchments. Upgrade timing/location is determined by system-wide engineering and regulatory planning, not local spills often. CSOs upstream of Lee Tunnel vs downstream / outside catchment in 2016, South-bank CSOs draining to Crossness vs others in 2012-16, Deephams catchment vs adjacent catchments in 2011-17, Thames Tideway Tunnel ongoing.

Historic hydraulic engineering design

Historical lock-in as the system and its pinch points for spillage are inherited.

- Deviations of Sewer Path from Shortest Distance

Chosen sewer path often deviated from the hydrological ideal to avoid expensive land acquisition, tunnelling, or crossing obstacles.

For a given CSO, the start point is the most upstream point of the historic sewer that feeds into the CSO and the end point is its outfall location. Then can calculate the ratio of actual length of the sewer line to the straight line (geodesic) distance between its start and end points. A higher ratio indicates a more circuitous, potentially cost-constrained route that could lead to suboptimal hydraulic performance (e.g., slower flow, more frequent blockages/surcharge) in the modern system, contributing to spill frequency. The deviation (Circuitousness > 1) was caused by Victorian engineers avoiding a hill, an expensive estate, or following a property line.

Flatter gradient, pumped section, pipe diameter, upstream population (London used census data), upstream number of major tributary junctions per mile of sewer (more on these in 2).

What can be constructed with existing data? Circuitousness, gradient, junction density, population pressure.

Circuitousness from sewer to CSO paths (historic maps OS/MBW and where missing would tend to follow natural/buried rivers and streams that can be also got from Ordnance Survey Open Rivers and OS OpenMap Local for waterlines/course and rivers/streams).

Gradient needs start and end points of sewer (OS Terrain 50 and LIDAR for topography)

Junction Density or junctions per km of CSO catchment (where two or more sewer lines meet)

Network Topology (if needed) can be arrived at if can identify main trucks versus branches, such as number of child pipes per parent pipe, total pipe length to catchment area.

Historical population needs old censuses (eg Bazalgette mentions using census tracts in MBW) and catchment boundaries to define the area whose runoff feeds each CSO

Catchment + Sewer path

Circuitousness = Length from head of catchment to CSO outfall versus Straight-line distance,

Gradient = Start point of CSO to End point of CSO / Length of path between these two points

+ Historical population  Capacity Pressure = Historical Population / Catchment Area

Expect that CSOs in the flat, lower Thames floodplain (e.g., Bermondsey) should have very low gradients. CSOs on the steeper slopes of North London (Hampstead) should have higher gradients.

- Hydraulic engineering pinch points

Hydraulic constraints are inherited from historical network design and affect spill probability, but are not salient to modern buyers except via spill risk.

Pipe capacity (diameter, invert level or depth, invert offsets, slope, material or roughness, trunk line class—truck sewer/secondary sewer/house lateral). Upstream population (and potential growth), upstream impervious area, hydraulic centrality (distance to treatment works, number of upstream nodes feeding into it, capital cost (depth to sewer, high groundwater levels, certain soft and permeable soil types, riverbank conditions including tidal conditions, diversions from other utilities and rainfall (and forecasts) were accounted for in designing the system.

What we can get from the archives?

Original proposed route (Bazalgette) v Actual As-Built route

Historic drawings to obtain pipe diameter and invert levels/gradients for the main trunk lines (like the Northern Outfall which is detailed here) and major tributaries, and to show the obstacles that cause the deviations (like a diversion to avoid a main Gas line in the drawings).

These are in the archives and are NOT digitized (as far as I know) and a sample is in the folder labelled “lma” that has photos of the drawings of the Northern Sewer Outfall of the Thames from the Bazalgette design.

Drawings provide high-precision spatial instruments or validation checks by looking for annotated diversions, changes in alignment at property lines, or notes explaining route choices to confirm that circuitousness is a cost-driven engineering compromise.

What variables can be extracted from the drawings?

Need to georeference from drawings (see Sheet 10). Then use longitudinal sections (elevation profiles along the sewer) and cross-sections (pipe shape and size) to construct high-precision instruments (see Sheets 14 and 6):

- Invert Levels (e.g., 12.76, 11.97, 9.10 on the drawings) – the exact elevation of the sewer bottom at specific chainages (distances).

- Gradients – either directly stated (e.g., 1 in 880, 1 in 1150) or calculable from invert levels and distances.

- Pipe Dimensions – from cross-sections: diameters (e.g., 4'0", 6'0"), egg-shaped dimensions (54" x 36"), and construction details (brick vs. concrete).

- Location Cues (e.g., street, landmark, sewer shape) to georeference.

- Chainage – distance from a starting point (e.g., 20 Miles 30 Chains), allowing precise location plotting.

Flatter gradients, notes like “tide affected”/”backwater”, pipe shape changes, smaller pipes are all good candidates for more spills.

Scaling up?

From the drawings, can get actual gradient (invert levels and stated gradients) and capacity pressure (pipe area v population), along with qualitative notes (like Gas Main for circuitousness) of the sewer system to examine the validity of constructed instruments and to optimise their weighting if needed.

If assume about 2-5 CSOs in London for every mile, then the NO (Hackney to Beckton) drawings in the folder cover about 2-5 high spill risk CSOs. Going by this, we can target a purposive sample of key hydraulic variation segments – flat, steep, major junction, circuitous, typical urban main sewer, extreme hydraulic headroom like a pumping station inlet. Possible candidates are – Beckton NO (flat), Hampstead (steep), Fleet Street (major junction), Crossness/Deptford/Abbey Mills (pumping station) that can be photographed to obtain the relevant variables.

Example of narrative

The historical engineering drawings reveal that sewer alignments were primarily determined by cost-minimisation in the face of physical constraints. For example, the Northern Outfall sewer deviates at Chainage 20.45 to avoid the deep excavation required under the Channelsea River (see Drawing 14), and again at Chainage 20.80 to follow a property boundary, as noted on the plan. These features—river crossings and parcel limits—are orthogonal to modern neighbourhood desirability but mechanically influenced the sewer's hydraulic performance. Our circuitousness instrument directly captures these inherited constraints.

The invert level of a pipe is the level taken from the bottom of the inside of the pipe as shown.

The level at the crown of the pipe = the invert level + internal diameter of the pipe + pipe wall thickness. It may be necessary to use this in calculations when level measurements are taken from the crown of a pipe.

Source: https://www.drainageconsultantsltd.co.uk/services/drainage-gradients-fal