Explanation for Sample of LMA photographs

Reference – labelling of LMA archive

Sheet 1

Title:

Main Drainage Metropolis – Northern Outfall Sewer – Barking Creek – Additional Reservoir

This is an engineering plan of an “additional reservoir” (settling basin) built as part of the Northern Outfall Sewer outfall works.

Key features you can see:

- “GENERAL PLAN” → this is a true plan view (map view), not a section.

- A regular grid across the basin:

  - This is setting-out gridwork used by engineers to position walls, floors, and piers accurately.

- Corner details (“North West Corner”, “South East Corner”, etc.):

  - These show brick/stone construction details, not geography.

- The long straight edge on the right:

  - This is the river wall / embankment line along Barking Creek / Thames edge.

- Notes like “Line of Ground taken with the Current level from the Specification”:

  - This tells you the drawing is construction-accurate, tied to site levels.

What it represents in modern terms:
A large rectangular settling or storage basin immediately adjacent to Barking Creek, just upstream of the Thames outfall — part of the final treatment/management of sewage before discharge.

Sheet 2

This one is especially useful for mapping.

What it shows:

- A keyed plan of land parcels at Barking Creek:

  - Marshes

  - embankments

  - river edge

  - plots acquired or affected by the works

- Barking Creek is explicitly named.

- River Thames clearly labelled.

- A north arrow (very important).

- Boundary lines with annotations (ownership / use / works).

This sheet is effectively a site location plan — it ties the engineering works to the surrounding landscape.

2) Where this is in today’s London (orientation)

These drawings sit in what is now roughly:

- Creekmouth / Barking Riverside / Beckton edge

- Where Barking Creek meets the Thames

- North bank of the Thames, east of today’s Barking town centre

Although the surface landscape has changed (dock infill, industrial estates, flood defences), the river geometry is still recognisable, which is why this can be georeferenced accurately.

3) How to place these on a contemporary London map (step-by-step)

The correct method: georeferencing

- If edges taper (camera angle), apply a perspective correction

  - This greatly improves accuracy later

C. Georeference using stable features (this is the key)

D. Check the fit

When done:

- The river edges on your historic drawing should sit neatly over modern river outlines

- Minor mismatch is normal; large warping means better control points are needed

Sheet 3: Sectional Plan – Additional Reservoir

- This is a horizontal slice through the structure, usually taken at a specific level (often near floor or springing level).

- You can see:

  - thick perimeter walls

  - internal piers/columns

  - repeated bays

  - central channels or openings

What the drawing is doing

- It shows the internal structural logic of the reservoir:

  - bay spacing

  - wall thicknesses

  - alignment of internal partitions

- The dotted/solid line conventions indicate different structural elements cut or seen below.

For CSO / spatial instruments

- ❌ Not directly mappable

- ✅ Very useful contextually:

  - tells you the internal capacity and subdivision

  - helps interpret how flow may have been throttled or stored

- Use it to:

  - interpret what kind of structure a mapped footprint actually represents

  - understand whether overflows would be point-based or distributed across bays

Sheet 4: Sectional Elevations (A–B / C–D)

What it is

- Vertical cross-sections through the reservoir

- You can see:

  - repeated arched chambers

  - roof structure

  - floor thickness

  - ground cover above

- Marked “Sectional Elevation at A–B”, etc.

Key information embedded

- Vertical relationships:

  - ground level

  - roof springing

  - invert levels (sometimes subtle)

- Hydraulic clues:

  - height of internal openings

  - potential surcharge depth before overflow

For CSO / spatial instruments

- ❌ Not georeferenceable

- ✅ Highly relevant hydraulically

- This is where you learn:

  - how much vertical storage existed

  - how close structures are to ground surface

- Use this to:

  - assign capacity / headroom attributes to spatial features

  - support modelling assumptions (e.g. “this reservoir could surcharge X metres before spill”)

Sheet 5: Top Plan – Additional Reservoir

This one matters a lot.

What it is

- Clearly labelled “TOP PLAN”

- This is a true plan view of the roof / upper structure

- Shows:

  - repeated roof vaults

  - access points / shafts

  - alignment relative to embankments

Why it’s important

- Unlike internal sectional plans, this:

  - reflects the external footprint

  - often matches what survives underground today

For CSO / spatial instruments

- ✅ YES – spatially useful

- This can be:

  - georeferenced

  - digitised as a polygon footprint

- Particularly useful for:

  - mapping storage infrastructure associated with overflow management

  - linking to later CSO interception works

Tip

- Combine this with your earlier General Plan sheet — they should align closely.

Sheet 6: Details – Penstocks, Manholes, Covers

What it is
A details sheet, showing:

- penstock valves

- vertical shafts

- manhole covers

- sections through walls with fittings

You can see notes like:

- “Working Penstock”

- ground levels

- pipe diameters

- wall thickness

This is gold for CSOs — but not spatially

For CSO / spatial instruments

- ❌ Not mappable directly

- ✅ Extremely important semantically

- This tells you:

  - where flow could be controlled

  - how overflow might be isolated or released

  - what kind of mechanical regulation existed

Use this to:

- classify a mapped point as:

  - overflow

  - control valve

  - access chamber

- add attributes like:

  - control_type = penstock

  - operation = manual

  - likely_role = overflow regulation

Summary Table (very important)

| Sheet | Type | Spatially mappable? | CSO relevance |
| --- | --- | --- | --- |
| A – Sectional Plan | Horizontal internal slice | ❌ No | Medium (capacity logic) |
| B – Sectional Elevations | Vertical sections | ❌ No | High (hydraulics, headroom) |
| C – Top Plan | External plan view | ✅ YES | Very high |
| D – Details | Components & fittings | ❌ No | Very high (control logic) |

Sheet 7: Mechanical details – penstocks & operating gear

What it is

- A mechanical engineering details sheet

- Shows:

  - large circular valve faces

  - operating spindles

  - gearing / handwheels

  - plan, elevation, and section of the same apparatus

- Notes reference:

  - “penstock”

  - guides

  - flanges

  - fixing details

What this tells you

- These are flow-control devices, not passive conduits.

- Penstocks like this were used to:

  - isolate chambers

  - regulate discharge

  - prevent backflow under tide/surcharge conditions

CSO relevance

- ❌ Not spatially mappable

- ✅ Very high functional relevance

For your spatial instrument:

- This drawing tells you that any mapped point associated with this structure is controllable, not a free overflow.

- Attribute ideas:

  - control = penstock

  - operation = manual

  - function = isolation / regulation

  - normal_state = closed/open (interpretive)

Sheet 8: Large vertical section through chamber & valve

What it is

- A long vertical section, probably through:

  - a chamber

  - an inlet/outlet conduit

  - the penstock assembly

- You can see:

  - massive masonry

  - floor slab

  - valve face

  - upstream/downstream voids

Important clues

- Heights and proportions show:

  - how much head was needed before flow passed

  - where surcharge would accumulate

- This is essentially a hydraulic behaviour diagram.

CSO relevance

- ❌ Not directly mappable

- ✅ Extremely important for CSO logic

This lets you:

- infer trigger levels (even if not numerically exact)

- distinguish between:

  - storage-first behaviour

  - overflow-once-full behaviour

Use it to:

- justify why an overflow occurs only after a certain capacity is exceeded.

Sheet 9: Elevation / section of circular conduit with control structures

What it is

- Another vertical section, but this time clearly focused on:

  - a large circular conduit

  - adjoining control chambers

- The repeated shading indicates:

  - waterway

  - masonry

  - ground

Why it matters

- This looks very much like a main sewer / outlet conduit immediately upstream or downstream of the reservoir.

CSO relevance

- ❌ Not mappable on its own

- ✅ High relevance for network interpretation

This drawing helps you:

- decide whether a mapped feature is:

  - mainline sewer

  - overflow branch

  - controlled outlet

- support assumptions like:

  - “this CSO discharges only once the main conduit is surcharged above X”

Sheet 10: Composite details – chamber, conduit profile, fittings

What it is

- A composite details sheet, combining:

  - conduit profiles

  - chamber cross-sections

  - small plan details

- Includes:

  - pipe shapes

  - fixing points

  - junction geometry

Why it exists

- This is a builder’s sheet: it tells contractors exactly how to assemble the system.

CSO relevance

- ❌ Not spatially mappable

- ✅ Very useful semantically

This sheet lets you:

- distinguish between:

  - access chambers

  - control chambers

  - flow-through conduits

- Avoid misclassifying features as CSOs when they are actually:

  - inspection access

  - structural transitions

CSO-focused summary (this batch)

| Sheet | Type | Spatial geometry? | CSO value |
| --- | --- | --- | --- |
| E | Penstock mechanical details | ❌ No | Very high (control logic) |
| F | Vertical chamber/valve section | ❌ No | Very high (trigger behaviour) |
| G | Main conduit + chamber section | ❌ No | High |
| H | Composite construction details | ❌ No | Medium–High |

Big-picture interpretation (important)

Across both batches so far, a clear pattern emerges:

- This is not a simple “overflow to river” site

- It is a controlled, staged system:

  - Flow enters main sewer

  - Storage is taken up in the reservoir

  - Penstocks regulate onward flow

  - Only under extreme conditions would uncontrolled discharge occur

For CSO mapping, this means:

- Many “outfall-looking” structures are actually regulated nodes

- You should model them as:

  - controlled discharge points

  - not automatic spill weirs

Sheet 11: Integrated section through reservoir + control works

What it is

- A large composite vertical section:

  - reservoir body

  - inlet/outlet conduits

  - penstock chambers

  - surrounding ground and embankments

- Includes small inset plans/details for specific fittings.

What this sheet is really doing

- This is the “how everything connects” drawing.

- It ties:

  - storage

  - control

  - conveyance
into a single hydraulic story.

CSO relevance

- ❌ Not directly mappable

- ✅ Extremely important for interpreting behaviour

Use this sheet to:

- understand sequence of operation during surcharge

- determine whether discharge to river is:

  - direct

  - throttled

  - delayed until storage is full

This drawing underpins logic, not geometry.

Sheet 12: Plan of Existing Reservoir (Barking Creek side)

This is a key spatial sheet.

What it is

- Title includes “PLAN OF EXISTING RESERVOIR”

- Clear plan view showing:

  - full reservoir footprint

  - internal subdivision

  - embankment line

  - Barking Creek explicitly labelled

Critical observations

- The reservoir shape is irregular, responding to:

  - creek alignment

  - embankment geometry

- This is not the same as the “additional reservoir” — it’s the earlier structure.

CSO / spatial use

- ✅ YES — directly mappable

- This should become:

  - a polygon feature in your GIS

- It is foundational for:

  - historic CSO storage capacity

  - understanding how later works expanded or modified overflow behaviour

This is one of the most important drawings you’ve shown so far.

Sheet 13: Sectional Plan of Existing Reservoir

Despite the title, this is still planimetric in the GIS sense.

What it is

- A plan view with section lines marked (A–B, C–D, etc.)

- Shows:

  - internal divisions

  - flow paths

  - relative arrangement of bays

Why it matters

- This drawing tells you:

  - how water moves across the reservoir

  - which areas fill first

  - how flow is distributed

CSO relevance

- ✅ Yes — spatially useful (secondary)

- Use it to:

  - enrich the polygon from Sheet J

  - subdivide storage into logical zones if needed

Think of this as:

“internal topology for a mapped footprint”

Sheet 14: New concrete counterforts (details)

What it is

- Structural details of reinforcement works

- Sections through:

  - embankments

  - walls

  - arches

- Focused on stability, not flow.

CSO relevance

- ❌ Not spatially mappable

- ❌ Low relevance to CSO logic

This sheet exists because:

- the reservoir was under stress

- strengthening was required

For your purposes:

- you can safely treat this as background engineering context

- reference it only if discussing structural longevity

CSO & spatial usefulness summary (this batch)

| Sheet | Description | Geometry? | CSO value |
| --- | --- | --- | --- |
| I | Integrated vertical section | ❌ No | Very high (system logic) |
| J | Plan of existing reservoir | ✅ YES | Very high (core geometry) |
| K | Sectional plan (plan view) | ✅ Yes (secondary) | High |
| L | Counterfort details | ❌ No | Low |

Big-picture milestone (important)

With this batch, you now have:

Spatially mappable assets

- Existing reservoir footprint

- Additional reservoir footprint

- River wall / embankment geometry

Behaviour-defining assets

- Penstocks

- Control chambers

- Vertical sections showing trigger levels

This is exactly what you need to build a CSO spatial instrument that distinguishes:

- passive overflow

- controlled discharge

- storage-first systems

Many modern CSO datasets flatten these distinctions — your work won’t.

Sheet 15: Existing Reservoir – New Penstocks (details)

What it is

- Title clearly refers to “Existing Reservoir – New Penstocks”

- This is a control retrofit drawing, not original construction.

- Shows:

  - penstock elevation

  - sectional plans through wall and valve seat

  - brackets, guides, and wall cut-throughs

Why this drawing exists

- The original reservoir did not have sufficient control

- New penstocks were added to:

  - better regulate discharge

  - isolate chambers

  - manage surcharge events

CSO relevance

- ❌ Not spatially mappable

- ✅ Very high functional relevance

For your CSO instrument, this drawing:

- confirms that discharge here is actively controlled

- supports modelling this as:

  - a regulated overflow node

  - not a passive spill

This is key evidence if you’re classifying CSOs by behaviour, not just location.

Sheet 16: General arrangement / site plan of reservoir & embankment

This is an important spatial sheet, even though it looks simple.

What it is

- A general arrangement plan showing:

  - reservoir footprint

  - embankments

  - adjacent yards / works

  - river-facing edge

- Less detailed internally, but very clear externally.

CSO / spatial use

- ✅ YES — spatially useful

- This is excellent for:

  - confirming footprint boundaries

  - cross-checking alignment between different plan sheets

  - anchoring the reservoir in relation to surrounding infrastructure

Think of this as:

a “sanity-check plan” for georeferencing

Sheet 17: Plan of concrete foundations

What it is

- Title refers to “Plan of Concrete Foundations”

- Shows:

  - pier/grid layout

  - foundation pads

  - structural repetition

What it’s for

- Structural load distribution

- Construction sequencing

CSO relevance

- ❌ Not spatially useful

- ❌ Low functional relevance

For your purposes:

- this does not change flow behaviour

- it does not define overflow points

You can safely exclude this from CSO instrumentation.

MASTER SYNTHESIS: MBW Barking Creek / Northern Outfall Sewer

Sheets 1–17: What they are + exactly how to use them

SHEETS TO BE USED AS SPATIAL ANCHORS (GEOMETRY)

These define where things are.

Sheet 1 – General Plan: Additional Reservoir

- ❗ Georeference this

- Use Barking Creek edge + Thames alignment as control points

- Digitise:

  - reservoir polygon

  - river wall line

Sheet 2 – Key / Location Plan (Barking Creek & Thames)

- ✅ Use this as your PRIMARY georeferencing base

- This sheet gives the best geographic truth

Cross-check with Sheets 3 and 10 if needed and 16 for alignment

SHEETS USED TO DERIVE HYDRAULIC & CSO BEHAVIOUR (ATTRIBUTES)

These define how things behave.

Sheet 4 – Sectional Plan (horizontal internal slice)

- Attribute: storage_type = compartmentalised

Sheet 5 – Sectional Elevations (A–B, C–D)

- ❗ Extract relative heights

- Attribute:

  - max_storage_depth

  - freeboard_estimate

Sheet 7 – Penstock Mechanical Details

- Attribute:

  - control_type = penstock

  - control_mode = manual

Sheet 8 – Large Vertical Section (valve & chamber)

- Attribute:

  - overflow_trigger = controlled

Sheet 9 – Main Conduit + Chamber Section

- Node classification

✅ Georeference FIRST:

Sheet 10

(Plan of Existing Reservoir)

- This polygon = available historic storage

- Storage ≈ reduced overflow likelihood

✅ Use: Sheet 5

(Sectional Elevations A–B / C–D)

Ignore everything except:

- Floor level

- Roof springing / crown

- Distance between them

You do not need absolute datum — only relative depth.

Extract

- effective_storage_depth ≈ X metres

(You can estimate visually if dimensions are hard to read; order-of-magnitude is enough.)

STEP 4 — CONFIRM CONTROL VS PASSIVE OVERFLOW

✅ Use: Sheet 7 or Sheet 13

(Penstock details / New penstocks to existing reservoir)

Single binary question

Is discharge actively controlled?

Answer from drawings:

- YES — penstocks present

Attribute

- controlled_discharge = true

STEP 5 — DERIVE CSO OVERFLOW LIKELIHOOD (SIMPLE, DEFENSIBLE)

You now have exactly three inputs:

| Factor | From | Meaning |
| --- | --- | --- |
| Storage present | Sheet 10 | Buffers storm inflow |
| Storage depth | Sheet 5 | Capacity magnitude |
| Active control | Sheet 7/13 | Prevents passive spill |

Simple rule-based classification

IF storage_present = yes

AND effective_storage_depth ≥ ~3 m

AND controlled_discharge = true

THEN overflow_likelihood = LOW

FINAL DELIVERABLE (MINIMAL)

Spatial

- One polygon: reservoir_existing

Attributes

storage_present = yes

effective_storage_depth = ~X m

controlled_discharge = yes

overflow_likelihood = LOW

confidence = high (MBW archival evidence)

That’s a very reasonable confusion — the distinction is subtle, and you’re right to stop and clarify. I’ll be very crisp and decisive here.

Georeference Sheet 10 first.
Use Sheet 2 only as a guide/check, not as the anchor.

Sheet 10 = BEST georeferencing anchor

What Sheet 10 has

- Large-scale plan view

- Clear, continuous river wall / embankment geometry

- Explicit label “Barking Creek”

- Geometry that still exists (or closely approximates) today

- Enough spatial extent to place multiple control points

Why that matters
Georeferencing works best when:

- features are large

- features are physically persistent

- distortions are low relative to feature size

Sheet 10 meets all three.

Sheet 2 = CONTEXT, not anchor

What Sheet 2 is good for

- Orientation (north arrow)

- Understanding land parcels and relationships

- Checking that you are not mirrored or rotated

- Confirming which side of the creek is which

What Sheet 2 is not good for

- Precise georeferencing

- Accurate scaling

- Digitising geometry

It is:

- small-scale

- cartographic

- more interpretive than geometric

👉 Think of Sheet 2 as a map legend made spatial, not a base layer.

The correct workflow (precise)

✅ Step 1 — Georeference Sheet 10

- CRS: EPSG:27700

- Control points:

  - Barking Creek bends

  - Creek–Thames junction

  - Straight embankment sections

- Output:

  - existing_reservoir_georef.tif

✅ Step 2 — Overlay Sheet 2 (ungrounded or loosely aligned)

- Use only to:

  - check orientation

  - confirm naming and relationships

- Do not digitise from Sheet 2

✅ Step 3 — Add measurements

- Vertical depth → Sheet 5

- Control logic → Sheet 7 / 13

One-sentence rule (to avoid future confusion)

If a sheet shows the river wall clearly and at large scale, georeference it.
If it explains where things are but not exactly how big they are, don’t.

Sheet 10 satisfies the first condition. Sheet 2 does not.

### Priority Ranking & Data to Extract

Focus on these drawings first. They contain the hydraulic profile data essential for calculating gradients and identifying bottlenecks.

| Drawing Number | Why It's Useful | What to Extract (Key Fields) |
| --- | --- | --- |
| 14 | The most important sheet. Shows a longitudinal profile (elevation view) of the sewer with invert levels, gradients, and chainages clearly marked. This is the core data for your gradient instrument. | - Chainage (e.g., "20 Miles 30 Chains") - Invert Level (e.g., "12.76", "11.97") at each point. - Gradient written as ratio (e.g., "1 in 880"). - Notes on location (e.g., "under Channelsea River"). |
| 6 | Shows a detailed cross-section with precise pipe dimensions and shape. This is critical for calculating flow capacity. | - Pipe shape & dimensions (e.g., "Egg 54" x 36"" or "4'0" Brick"). - Construction material (Brick, Concrete). |
| 9 & 10 | These show plan views with junctions and tributaries. Essential for understanding network topology and identifying convergence points (which increase spill risk). | - Junction points (where sewers merge). - Pipe diameter changes at junctions. - Street names or landmarks for georeferencing. |
| 13 | Another profile view, likely for a different section. Provides continuity and more gradient data points. | Same as Drawing 14: Chainage, Invert Levels, Gradient. |
| 5 | Appears to show a structural detail at a special junction or outfall. Useful for understanding unique hydraulic features. | - Special structure type (e.g., weir, outfall chamber). - Dimensions of the structure. |